Circuit Diagram:

Figure: Interfacing the LED blinking using PIC16F877A

Source Code:

void main(){ TRISB = 0x00; portb = 0x00; while(1){ portb.f0 = 0xff; delay_ms(500); portb.f0 = 0x00; delay_ms(500); } }

Circuit Diagram:

Figure: Interfacing the 7 segment display using PIC16F877A

Source Code:

char arraCC[] = {0x3F,0x06,0x5B,0x4F,0x66,0x6D,0x7D,0x07,0x7F,0x6F}; char arraCA[] = {0xC0,0xF9,0xA4,0xB0,0x99,0x92,0x82,0xF8,0x80,0x90}; void main() { int i = 0; TRISD = 0x00; //set all pincs of port d TRISB = 0x00; for(i = 9; i >= 0; i--){ portb = arraCA[i]; delay_ms(1000); if(i == 0) i = 10; //again restart } }

Circuit Diagram:

Figure: Reading the ADC value in virtual terminal using PIC16F877A

Source Code:

int valADC, valADC1; char x[4]; void main() { UART1_Init(9600); ADC_Init(); while(1) { valADC = ADC_Read(0); valADC1 = ADC_Read(1); IntToStr(valADC,x); UART1_Write_Text("Analog value ="); UART1_Write_Text(x); UART1_Write_Text(" "); IntToStr(valADC1,x); UART1_Write_Text("Analog value ="); UART1_Write_Text(x); UART1_Write(13); Delay_ms(1000); } }

Circuit Diagram:

Figure: Interfacing the 2-digit 7-segment multiplexing using PIC16F877A

Source Code:

char arraCC[] = {0x3F,0x06,0x5B,0x4F,0x66,0x6D,0x7D,0x07,0x7F,0x6F}; void main() { int mod = 0, res = 0, i = 0, k = 0, state = 1; TRISB = 0x00; portb = 0x00; TRISC = 0x00; portc = 0x00; while(1) { for(i = 0; i <= 22; i++) { res = i/10; mod = i%10; for(k = 0; k < 50; k++) { portc.f0 = 0x00; // active power for digit left portb = arraCC[res]; // provide data for two digit delay_ms(10); portc.f0 = 0xff; // Deactive power for digit right portc.f1 = 0x00; // active power for digit right portb = arraCC[mod]; delay_ms(10); portc.f1 = 0xff; // deactive power for digit left } } } }

Circuit Diagram:

Figure: Interfacing the LED with push-button using PIC16F877A

Source Code:

void main(){ TRISB = 0x00; TRISC = 0Xff; portb = 0x00; while(1){ if(portc.f0 == 0xff) portb.f0 = 0xff; else portb.f0 = 0x00; } }

Circuit Diagram:

Figure: Interfacing the Dot matrix using PIC16F877A

Source Code:

| void main() { TRISC = 0x00; TRISD = 0x00; while(1) { PORTD = 0x80; PORTC = 0x00; delay_ms(5); PORTD = 0x40; PORTC = 0xff; delay_ms(5); PORTD = 0x20; PORTC = 0xff; delay_ms(5); PORTD = 0x10; PORTC = 0x18; delay_ms(5); | PORTD = 0x08; PORTC = 0x18; delay_ms(5); PORTD = 0x04; PORTC = 0xff; delay_ms(5); PORTD = 0x02; PORTC = 0xff; delay_ms(5); PORTD = 0x01; PORTC = 0x00; delay_ms(5); } } |
| --- | --- |

Circuit Diagram:

Figure: Interfacing the LM-35 temperature sensor using PIC16F877A

Source Code:

| // LCD module connections sbit LCD_RS at RB2_bit; sbit LCD_EN at RB3_bit; sbit LCD_D4 at RB4_bit; sbit LCD_D5 at RB5_bit; sbit LCD_D6 at RB6_bit; sbit LCD_D7 at RB7_bit; sbit LCD_RS_Direction at TRISB2_bit; sbit LCD_EN_Direction at TRISB3_bit; sbit LCD_D4_Direction at TRISB4_bit; sbit LCD_D5_Direction at TRISB5_bit; sbit LCD_D6_Direction at TRISB6_bit; sbit LCD_D7_Direction at TRISB7_bit; // End LCD module connections char cel[20], temp[40], far[20]; int i, k, valADC, farhen; void main(){ UART1_Init(9600); // Initialize UART module with baud rate 9600 Lcd_Init(); // Initialize LCD | Lcd_Cmd(_LCD_CLEAR); // Clear display Lcd_Cmd(_LCD_CURSOR_OFF); // Cursor off i = 0; while(1) { //Read ADC value in RA2 valADC = ADC_Read(0); farhen = (valADC * 9/5) + 32; //Convert into string/char array IntToStr(valADC,cel); IntToStr(farhen,far); Lcd_Out(1,1,"Celsius:"); Lcd_Out(1,10,cel); // Write text in first row Lcd_Out(2,1,"Fahrenheit:"); Lcd_Out(2,10,far); } } |
| --- | --- |

Circuit Diagram:

Figure: Interfacing the mechanical relay using PIC16F877A

Source Code:

void main() { TRISB = 0x00; portb = 0x00; while(1) { portb.f0 = 1; delay_ms(1000); portb.f0 = 0; delay_ms(1000); } }

Circuit Diagram:

Figure: Interfacing PWM for DC motor speed controlling using PIC16F877A

Source Code:

| void main() { unsigned short duty = 0; TRISB = 0x00; TRISC = 0x00; TRISD = 0xff; portb.f0 = 0xff; // initialize portb.f1 = 0x00; PWM1_Init(1000); // Initialize PWM PWM1_Start(); // start PWM PWM1_Set_Duty(duty); // set current duty for PWM1 while(1) { // Increment Button Activities if(portd.f0 ==1) { delay_ms(200); if(duty <= 240) { if(portd.f0 == 1) { | duty = duty +10; PWM1_Set_Duty(duty); } } } // Decrement Button Activities if(portd.f1 == 1) { delay_ms(200); if(duty >= 10) { if(portd.f1 == 1) { duty = duty - 1; PWM1_Set_Duty(duty); } } } } } |
| --- | --- |

Circuit Diagram:

Figure: Interfacing servo using PIC16F877A

Source Code:

| void rotateLeft90(); void rotateZero(); void rotateRight90(); int i; void main() { TRISB = 0X00; TRISC = 0XFF; portb = 0x00; rotateZero(); while(1) { if(portc.f0 == 0xff) { rotateLeft90(); } if(portc.f1 == 0xff) { rotateZero(); } if(portc.f2 == 0xff) { rotateRight90(); } } } void rotateLeft90() { | for(i=0;i<50;i++) { portb.f0=1; delay_us(800); portb.f0=0; delay_us(19200); } } void rotateZero() { for(i=0;i<50;i++) { portb.f0=1; delay_us(1500); portb.f0=0; delay_us(18500); } } void rotateRight90() { for(i=0;i<50;i++) { portb.f0=1; delay_us(2200); portb.f0=0; delay_us(17800); } } |
| --- | --- |

Circuit Diagram:

Figure: Interfacing the stepper motor using PIC16F877A

Source Code:

void main() { TRISD = 0X00; portd = 0x00; while(1) { portd = 0b00000011; delay_ms(50); portd = 0b00000110; delay_ms(50); portd = 0b00001100; delay_ms(50); portd = 0b00001001; delay_ms(50); } }